Mobile Phone Surveillance User Mannual

# Catalogue

Catalogue	0

1 Symbian System	1

1.1Brief Introduction	1

1.2 Installation and Operation Instructions	1

2 Windows mobile System	4

2.1 Brief Introductions	4

2.2 Installations and Operations Instructions	5

3 iPhone	10

3.1Brief Instructions	10

3.2 Installations and Operation instructions	10

4 Blackberry	14

4.1 Brief Instructions	14

4.2 Installation and Operation Instructions	14

5 Android	21

5.1Brief Instructions	21

5.2 Installation and Operation Instructions	21

# 1 Symbian System

## 1.1Brief Introduction

The main functions and characteristics of the Mobile Phone(Symbian System) are as below:

Real time viewing video function

PTZ control function(front end is the controllable dome camera)

Prerequisite

The mobile phone  should support  9.1 Version Symbian System(just as said the Third and Fifth Generation of the Symbian System) such as Nokia 6120c, Nokiae71 and Nokia5800( all the mobile phones with a resolution of 240*320,320*240 in the third generation and a resolution of 360*640,640*360 in the fifth generation.

## 1.2 Installation and Operation Instructions

- 1) Open the 'Symbian' folder of the CD disk files,then transfer the files(KMEYE_3rd sisx or KMEYE_5rd sisx) to the mobile phone for the installation on the phones

Note:KMEYE_3rd.sisx support Symbian system of the Third Generation,KMEYE_5th.sisx support Symbian system of the Fifth Generation

There will be the programme icon on the mobile phones, try to find the KMEye under the designated root catalogue,as shown in the following image:

- 2) Click KMEye programme icon to enter into the interface as shown in the following image:

The corresponding functions of the Buttons:

| Function Key | Name | Descriptions of the functions |
| --- | --- | --- |
|  | PLAY | Connect the device and play the video |
|  | Screenshot | Screenshot of the available images(Save the screenshot image to the defaulted photo root of the mobile phones) |
|  | Setting | Setting of the IP address, dynamic domain name,Port number,user,password and device names |
|  | Next Channels | Select to the next Channels |
|  | Exit | Exit |
|  | Full Screen | Full Screen Viewing |
|  | Channel Selection | Select to play the video of one channel |
|  | PTZ Control | To control the PTZ(UP,DOWN,RIGHT,LEFT) |
|  | Focus,Zoom,Iris | To widen and Truncate, adept the focus distance |

- 3) If using the mobile viewing for the first time or any need to modify the parameters,please click 'Setting' button to enter into the interface of parameter setting as shown in the following image:

Note:

Server Address: The IP address or Dynamic domain name of the DVR IE

Server Port:The server port on the DVR

Device Name:the alias to differentiate the different devices

User ID: The same user  name with the name set on IE

Password:The same password  with the password set on IE

alias:create the corresponding alias.So the alias will appear the alias on the interface

Open the menu to enter into the list of the history record, or click the PLAY&BACK button to play,then the setting will be saved. The setting will not be saved if clicking BACK directly.

Select one of the history records and open the menu,then click OK to see the viewing

Deleting one history record

Select the record file to be deleted and open the menu,then click the delete button to delete the file

4) Video Display Format

The switching between regular format and full screen format

Regular Format

When extending the mobile phone,the video will be played with full screen automatically.

5)Channel Switching

Select any of the channels,then the programme will open the corresponding channel and also play the record video automatically. The defaulted programme setting is 1-4 channel, when you click the NEXT option, the channel number will be switched between 5-8,8-12,12-16 accordingly.

# 2 Windows mobile System

## 2.1 Brief Introductions

The main functions and characteristics of the Mobile Phone(Windows Mobile System) are as below:

Real time viewing video function

PTZ control function(front end is the controllable dome camera)

Prerequisite

The mobile phone should support the Windows Mobile 2003 version or updated versions(only touch screen version supported for the present)

## 2.2 Installations and Operations Instructions

- 1) Open the 'Windows  Mobile' folder of the CD disk files,then transfer the files(KMEyesetup.cab) to the mobile phone for the installation on the phones

- 2) Try to find the KMEyesetup.cab under the designated file root, double-click it to install. After the installation, there will be the icons as following image:

- 3) Click KMEye Icon to enter into the interface as following image:

The corresponding functions of the Buttons:

| Function Key | Name | Function Description |
| --- | --- | --- |
|  | Connect and Play | Connect the device and begin to play the video |
|  | Setting | Setting of the IP address, dynamic domain name,Port number,user,password and device names |
|  | Stop | Stop the Video Playing |
|  | Channel Selection | Select to play one video of one channel on the device |
|  | PTZ Control | To control the PTZ(UP,DOWN,RIGHT,LEFT) |
|  | Zoom | Zoom |
|  | Focus | Change the focus distance |
|  | Iris | Change the Iris |
|  | Screenshot | Screenshot of the available image,the screenshot file is saved under the installation root as follows: |

- 4) If using the mobile viewing for the first time or any need to modify the parameters,please click 'Setting' button to enter into the interface of parameter setting as shown in the following image:

Server Address: The IP address or Dynamic domain name of the DVR IE

Server Port:The server port on the DVR

Device Name:the alias to differentiate the different devices

User ID: The same user  name with the name set on IE

Password:The same password  with the password set on IE

alias:create the corresponding alias.So the alias will appear the alias on the interface

Click History button to enter into the history record interface

Select the record to be deleted,then press the Delete button to delete the record

Select one of the records and press OK button and then back to Setting interface

Then press OK button to enter into the connecting interface

Enter into the PLAY interface after the successful connecting

5)Video Display Mode

The switch between Regular Mode and Full Screen Mode

Regular Mode

Click Disconnect Button to stop the playing

Full Screen Mode

Click the Screen with the finger or Mobile screen pen to switch into the full screen play

Click the Screen with the finger or Mobile screen pen when full screen playing,the screen will be switched into the regular play mode

6）Channel Switching

Click to select the corresponding Channels,the program will open the corresponding channel and play hte record video automatically.

# 3 iPhone

## 3.1Brief Instructions

The main functions and characteristics of the Mobile Phone(Iphone) are as below:

Real time viewing video function

PTZ control function(front end is the controllable dome camera)

Prerequisite

The mobile phone should support the iOS 3.0 version or updated versions(Including Iphone and Ipod touch)

## 3.2 Installations and Operation instructions

1) To operate the APP Store Programme on IPHONE and switch into the search label web,then enter KMEye in the search column to find the corresponding programme installation files package. After that,click the file to install as shown in the following image:

- 2)Click KMEye Programme Icon to enter into the following image:

The corresponding functions of the Buttons:

| Function Key | Name | Function Description |
| --- | --- | --- |
|  | Play | Connect the device and play the video |
|  | Screen Shot | Screenshot of the available images(Save the screenshot image to the defaulted photo root of the mobile phones) |
|  | Setting | Setting of the IP address, dynamic domain name,Port number,user,password and device names Select to the next Channels |
|  | Next Group | Selection of next group |
|  | About | About Info |
|  | Channel Selection | select to play the video of one channel |
|  | PTZ Control | To control the PTZ(UP,DOWN,RIGHT,LEFT) |
|  | ZOOM | ZOOM |
|  | Focus | Change the focus distance |
|  | Iris | Change the Iris |

- 3) If using the mobile viewing for the first time or any need to modify the parameters,please click 'Setting' button to enter into the interface of parameter setting as shown in the following image:

Server Address: The IP address or Dynamic domain name of the DVR IE

Server Port:The server port on the DVR

Device Name:the alias to differentiate the different devices

User ID: The same user  name with the name set on IE

Password:The same password  with the password set on IE

Click History button to enter into the history record interface

Delete of one history record

Click 'Edit' button and click the red round button in one of the records as shown in the following image, then click Delete button

4)Video Display Mode

Switch between Regular Mode and Full Screen Mode

Regular Mode

When lying the mobile phone,the video will automatically be displayed with full screen.

- Channel Switching

Select any of the channels,then the programme will open the corresponding channel and also play the record video automatically. The defaulted programme setting is 1-4 channel, when you click the NEXT option, the channel number will be switched between 5-8,8-12,12-16 accordingly.

# 4 Blackberry

## 4.1 Brief Instructions

The main functions and characteristics of the Mobile Phone(Iphone) are as below:

Real time viewing video function

PTZ control function(front end is the controllable dome camera)

Prerequisite

The mobile phone  should support  4.6 Version Blackberry System such as Blackberry 8900, Blackberry 9000, Blackberry 9700  and  Blackberry 9630 with a resolution of 480*360 and 480*320.

## 4.2 Installation and Operation Instructions

- 1) Open the 'Blackberry' folder of the CD disk files,then transfer the files(KMEYE.cod or KMEYE.alx) to the mobile phone for the installation on the phones

Click the Options on the Mobile Phones to enter into the following interface:

Click 'application' option to find the KMEye,then click 'Authority Edit' to set all the authority as 'allow'

- 2) Click KMEye program icon. After that, enter into the initialization interface as shown in the following image:

Server Address: The IP address or Dynamic domain name of the DVR IE

Server Port:The server port on the DVR

Device Name:the alias to differentiate the different devices

User ID: The same user  name with the name set on IE

Password:The same password  with the password set on IE

Network Type:Select the desired web

The corresponding functions of the Buttons:

| Function Key | Name | Function Description |
| --- | --- | --- |
|  | Login | Enter into the PLAY main interface,then the program is connected to the device and begin to play the video |
|  | Cancel | Exit to the program |
|  | History | Enter into the history record interface |

Click LOGIN to enter into the PLAY main interface

The corresponding functions of the Buttons:

| Function Key | Name | Function Description |
| --- | --- | --- |
|  | PLAY | Connect the device and play the video |
|  | FULL SCREEN | FULL SCREEN PLAY |
|  | Screenshot | Screenshot of the available images(Save the screenshot image to the defaulted photo root of the mobile phones) |
|  | Setting | Setting of the IP address, dynamic domain name,Port number,user,password and device names |
|  | Next Group | Select to the next group |
|  | About | About Info |
|  | Channel Selection | select to play the video of one channel |
|  | PTZ Control | To control the PTZ(UP,DOWN,RIGHT,LEFT) |
|  | ZOOM | ZOOM |
|  | Focus | Change the focus distance |
|  | Iris | Change the Iris |

Click Cancel and exit the program as shown in the following image:

- 3) Click 'History Record' to enter into the history record list as shown in the following image:

- Click the menu button to pop-up the menu

- The corresponding functions of the menu are as following

| Function Key | Name | Function Description |
| --- | --- | --- |
| open | PLAY | Connect the device and play the video |
| edit | EDIT | EDIT RECORD |
| Clear savePath record | CLEAR | Clear savePath record |
| New Device | New Device | New added record |
| Delete | delete | delete the records |
| Delete All Records | Delete All Records | Delete All Records |
| Switch Application | Switch Application | Switch Application |
| Close | Close | Close the program |

4)Video Display Mode

The switch between the regular mode and full-screen mode

Regular Mode

Full Screen Mode

# 5 Android

## 5.1Brief Instructions

The main functions and characteristics of the Mobile Phone(Android) are as below:

Real time viewing video function

PTZ control function(front end is the controllable dome camera)

Prerequisite

The mobile phone  should support  1.5 Version Android System such as HTC G1,HTC Magic,HTC Hero,HTC G5 and so on .

## 5.2 Installation and Operation Instructions

- 1) Open the 'Android' folder of the CD disk files,then transfer the files(KMEye.apk) to the mobile phone for the installation on the phones

- 2)Try to find the KMEye.apk under the designated root and click the Ultilization Program to finish the installation

- 3) Click the KMEye Icon to enter into the following interface:

The corresponding functions of the Buttons:

| Function Key | Name | Function Description |
| --- | --- | --- |
|  | PLAY | Connect the device and play the video |
|  | Screenshot | Screenshot of the available images(Save the screenshot image to the defaulted photo root of the mobile phones) |
|  | Setting | Setting of the IP address, dynamic domain name,Port number,user,password and device names Select to the next Channels |
|  | Next Group | Select to the next group |
|  | About | About Info |
|  | Channel Selection | select to play the video of one channel |
|  | PTZ Control | To control the PTZ(UP,DOWN,RIGHT,LEFT) |
|  | ZOOM | ZOOM |
|  | Focus | Change the distance of focus |
|  | IRIS | Change the IRIS |

- 4) If using the mobile viewing for the first time or any need to modify the parameters,please click 'Setting' button to enter into the interface of parameter setting as shown in the following image:

Server Address: The IP address or Dynamic domain name of the DVR IE

Server Port:The server port on the DVR

Device Name:the alias to differentiate the different devices

User ID: The same user  name with the name set on IE

Password:The same password  with the password set on IE

is the Back button

is referred as the entry of the history record interface

Select one of the record in the history record list to watch the video

Delete of one history record

Select the record to be deleted and click it to pop up a selection column.select delete option to delete the history record

5)Video Display Mode

The switch of the Regular Mode and Full Screen Mode

Regular Mode

When the mobile phone is lying,the video is played with full screen automatically

Full Screen Mode

6)Channel Switch

Select any of the channels,then the programme will open the corresponding channel and also play the record video automatically. The defaulted programme setting is 1-4 channel, when you click the NEXT option, the channel number will be switched between 5-8,8-12,12-16 accordingly.